Practical 1

import numpy as np

# Spreading codes

c1 = [1, 1, 1, 1]

c2 = [1, -1, 1, -1]

c3 = [1, 1, -1, -1]

c4 = [1, -1, -1, 1]

rc = []

print("Enter the data bits:")

d1 = int(input("Enter D1: "))

d2 = int(input("Enter D2: "))

d3 = int(input("Enter D3: "))

d4 = int(input("Enter D4: "))

# Multiply data with spreading codes

r1 = np.multiply(c1, d1)

r2 = np.multiply(c2, d2)

r3 = np.multiply(c3, d3)

r4 = np.multiply(c4, d4)

# Resultant channel signal

resultant_channel = r1 + r2 + r3 + r4

print("Resultant Channel:", resultant_channel)

# Select receiver code

Channel = int(input("Enter the station to listen for (C1=1, C2=2, C3=3, C4=4): "))

if Channel == 1:

rc = c1

elif Channel == 2:

rc = c2

elif Channel == 3:

rc = c3

elif Channel == 4:

rc = c4

else:

print("Invalid channel selected")

exit()

# Inner product calculation

inner_product = np.multiply(resultant_channel, rc)

print("Inner Product:", inner_product)

res1 = sum(inner_product)

data = res1 / len(inner_product)

print("Data bit that was sent:", data)

OUTPUT

Practical 4

clear

N = 10^6; % number of bits or symbols

rand('seed',100); % initializing the rand() function

randn('seed',200); % initializing the randn() function

% Transmitter

ip = rand(1,N) > 0.5; % generating 0,1 with equal probability

s = 2*ip - 1; % BPSK modulation 0 -> -1; 1 -> 1

n = 1/sqrt(2) * [randn(1,N) + j*randn(1,N)]; % white gaussian noise, 0dB variance

Eb_N0_dB = [-3:10]; % multiple Eb/N0 values

for ii = 1:length(Eb_N0_dB)

% Noise addition

y = s + 10^(-Eb_N0_dB(ii)/20) * n; % additive white gaussian noise

% receiver - hard decision decoding

ipHat = real(y) > 0;

% counting the errors

nErr(ii) = size(find([ip - ipHat]),2);

end

simBer = nErr / N; % simulated ber

theoryBer = 0.5 * erfc(sqrt(10.^(Eb_N0_dB/10))); % theoretical ber

% plot

close all

figure

semilogy(Eb_N0_dB, theoryBer, 'b.-');

hold on

semilogy(Eb_N0_dB, simBer, 'mx-');

axis([-3 10 10^-5 0.5])

grid on

legend('theory', 'simulation');

xlabel('Eb/No, dB');

ylabel('Bit Error Rate');

title('Bit error probability curve for BPSK modulation');

OUTPUT

Practical 5

Network consisting of router, switch and end devices

Configuring the router as a DHCP Server

Assigning IP Address using DHCP

DHCP Server Practical:

en

conf t

int fa0/0

ip add 192.168.1.1 255.255.255.0

no sh

exit

ip dhcp pool pool1

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

Practical 8

Configuring FTP on the server:

Sending files from Laptop to server using FTP

FTP-

ftp 192.168.1.1

username:cisco

password:cisco

dir

put test.txt

dir

Sending files from Laptop to server using FTP using python

├── server.py

├── client.py

├── abc.txt

-----------------------------------------------------------------

|                            SERVER.PY                          |

-----------------------------------------------------------------

import socket

HOST = "localhost"

PORT = 5000

FORMAT = "utf-8"

# Create TCP socket

server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

# Bind IP and port

server.bind((HOST, PORT))

# Wait for client

server.listen(1)

print("Waiting for connection...")

# Accept client

conn, addr = server.accept()

print("Connected by:", addr)

# Receive filename

filename = conn.recv(1024).decode(FORMAT)

print("Receiving file:", filename)

# Send acknowledgement

conn.send("Filename received".encode(FORMAT))

# Open file to save data

file = open(filename, "wb")

# Receive file data continuously

while True:

data = conn.recv(1024)

if not data:

break

file.write(data)

print("File received successfully")

# Close everything

file.close()

conn.close()

server.close()

-----------------------------------------------------------------

|                            CLIENT.PY                          |

-----------------------------------------------------------------

import socket

HOST = "localhost"

PORT = 5000

FORMAT = "utf-8"

FILE_NAME = "abc.txt"

# Create TCP socket

client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

# Connect to server

client.connect((HOST, PORT))

# Send filename

client.send(FILE_NAME.encode(FORMAT))

# Receive acknowledgement

msg = client.recv(1024).decode(FORMAT)

print(msg)

# Open file

file = open(FILE_NAME, "rb")

# Send file data continuously

while True:

data = file.read(1024)

if not data:

break

client.send(data)

print("File sent successfully")

# Close everything

file.close()

client.close()